Lecture 2

CE260

G. Hoag/Spring 2000

# Units of Expression

- Molarity=molesunit Vol⇒1M=1 moleL

- Molal=moles1000g Solvent⇒1m=1 mole1000g

    - For dilute aqueous solutions in H2O 1M  1m

- Mole Fraction=Xi=ni1kn=mole of a compound (i)total of moles of k substances

  - Mass Concentration

    - Example ppm, ppb, ppt

- ppm=mass substancemass solution=1 g Cl106 g H2O

    - For dilute aqueous solution in H2O 1mg/L = 1ppm

## Expression of concentration

    - e.g. [NH3] also [NH3-N] molar concentration of N in NH3 form (equivalent conc.)

    - Recall mass balance on dissolved nitrogen

CTin=[NO3−−N]+[NO2−−N]+[NH3−N]+[NH4+−N]

  - Some definitions

    - Molecules – groups of atoms bonded together

    - Ions – charged species

    - Valence - # of electrons a species has lost or gained e.g. Al3+, Cl-

    - Covalent bon – electrons are shared among atoms

      - When like atoms are covalently bonded e.g. O2 electrons are equally shared

      - When unlike atoms are covalently bonded e.g. NH3 one atom more strongly attracts electrons from the other atom causing a negative charge distribution different from + charge distribution – the compound is polar.

        - Polar bonds result in the formation of secondary bonds (influence – volatility, viscosity, solubility, etc.)

### Oxidation Number

  - f(# electrons associated)

  - Rules for assigning an oxidation #

    -  of oxidation number of a neutral compound is 0, otherwise the sum must equal the charge of the ionic species

    - oxidation number of all elements in their free state is 0

    - alkali metals (1st column of the periodic table) oxidation state = +1

alkali earth metal (2nd column of the periodic table) oxidation state = +2

### oxidation state of O is –2 except for peroxide then –1

### hydrogen has an oxidation state of +1 except in metal hydrides = -1 e.g. NaH

### When Mass Balancing a Reaction

  - Mass must be conserved

  - Charge must be conserved

  - Balancing the reactions

    - MnO2 + 4HCl  MnCl2 + Cl2 + 2H2O

      - Cl2 no charge species

    - MnO4- + H2O + e-  MnO2 + OH-

      - Step 1 balance atoms

    - MnO4- + 2H2O + e-  MnO2 + 4OH-

      - Step 2 balance the charge

      - O = -2, Mn = +7 to get –4 on left need 3e-

    - MnO4- + 2H2O + 3e-  MnO2 + 4OH-

    - Note: I would like to denote phase of species

    - ag(aqueous), g(gas), s(solid)

### Oxidation Reduction Reactions “Redox”

  - Involve electron transfer

    - One compound gives up electron (oxidized or reducing agent) - can write rxn

    - One compound gains an electron (reduced or oxidizing agent) – can write rxn

    - These separate rxns are called half reactions

    - In the overall reaction e- do not always appear – convention for writing half reactions is that they are written as reduction equations

    - e.g. Remember   MnO4- + 2H2O + 3e-  MnO2 + 4OH- (rxn #23, pg. 11)

    - 1/3 MnO4- + 4/3 H+ +e- = 1/3 MnO2 +2/3 H2O (this is a half reaction)

      - permanganate is reduced (it is an oxidant)

    - (rxn #14) Fe3+ + e- = Fe2+ reverse Fe2+ = Fe3+ + e-

    - Redox reaction

    - 1/3 MnO4- + Fe2+ + 4/3 H+ = 1/3 MnO2 +2/3 H2O + Fe3+

  - We can see how a Redox reaction is comprised of the reduction half rxn and the oxidation half reaction

    - Fe2+ is oxidized “losses electrons”

    - KMnO4 is reduced “gains electrons”

### Equilibrium

  - Recall that elementary reactions (rxn order can  be determined from stoichiometry) may be mono, di and sometimes tri- molecular rxns

#### AA +bB  cC + dD at equilibrium

    - Equilibrium expression is:

Keq=[C]c[D]d[A]a[B]b

    - Time to reach equilibrium is another matter and is a function of the type of reaction

    - We also recall equilibrium constant K= f(temperature)

      - VF = rate of forward reaction = k1 [A]a [B]b

      - Vr = rate of reverse reaction = k2 [C]c [D]d

      - At equilibrium Vf = Vr

      - Therefore k1 [A]a [B]b = k2 [C]c [D]d

Keq=k1k2=[C]c[D]d[A]a[B]b

- Conductivity and Ionic Strength

  - Concentration and charge of ions in solution affect the characteristics of the solutions

  - Conductance

    - Ability of solution to carry a current

    - Recall ohms law

#### E = iR

    - Where

      - E = voltage drop across electrodes

      - i =  current in amps

      - R = resistance in Ohm’s

    - Resistance is  to separation of the electrodes and inversely  to the surface area of the electrode

#### R = l/A

    - Where

      - R = resistance (Ohm)

      -  = specific resistance of sol’n (Ohm-cm)

      - l = distance between electrode (cm)

      - A = surface area of electrode (cm2)

    - Specific conductance – related to the sum of all charge carriers both + and – ions

      - Inverse of specific resistance K= 1/ (mhos/cm)

  - Equivalent weight

Eq. weight=GMWcharge on species

    - Ideally one equivalent weight of any substance would have the same specific conductance, so a 1N solution contains 1 equiv. wt/L and K is related to equivalent conductance () of a compound

K=kN1000λ

    - Where: k= adjustment factor (ideal solution k = 1)

    - Specific conductance is used to estimate TDS

    - Standard methods 1992

TDS=kκ

      -  = umhos/cm

      - k varies from 0.55 to 0.90

    - Ionic strength =  =   of all charged species in solution

μ=12CiZi2

    - Where:

      - Ci = molar concentration of ith ion

      - Zi = charge of Ith ion

    - Langlre (1936) developed correlation

      -  = 2.5*10-5 (TDS mg/l)

    - Russell 1976

      -  = 1.6*10-5 K (umhos/cm)

### Chemical Kinetics

dCAdt=±k CAa CBb…CNn

  - k is a rate constant

    - If k = + then production

    - If k = - then removal

  - mA  Production

d[A]dt=−k[A]n

    - n is the order of the reaction

    - zero order, 1st order, and 2nd order respectively

d[A]dt=−k, d[A]dt=−k[A], d[A]dt=−k[A]2

  - Retardant – rate change  with time

d[A]dt=±k1+αt[A]

    -  = characteristic reaction constant

    - autocatalytic – reaction accelerates spontaneously

    - catalysis – accelerates a reaction but is not changed in composition as a result of the reaction

    - Arrhenius found that chemical reaction rates (k) inc. with increasing temperature

k=A e(Ea/RT)

    - Linearized

ln k=ln A−EaRTy=b−mx

1/T

      - Ea = f(rxn mechanism type)

    - Can rearrange Arrhenius

kT2=kt1θ(T2−T1)

    - Where:

      - kT1 = reaction rate at T1

      - kT2  = reaction rate at T2

      -  = constant and f(type of reaction)

  - Gas laws

    - Boyles law (pV is a constant)

pV=k

p = pressure

ln k

Ea/RT

      - V = volume

    - Charles law

VT=k

      - V = volume at a fixed pressure

      - T = temp (K)

    - Ideal gas law

pV=nRT

    - Dalton’s law

PT=P1+P2+…Pn partial pressuresso Pi=ninTPT, or Pi=ViVT

    - Henry’s law

KH=G(aq)G(g)G(g)⇔G(aq)

K=f(Temp) and compound

KH O2 @ 0°C=69.6mgL−atm in H2OKH O2 @ 20°C=39.3mgL−atm in H2O

error in book Table 1.4 mg/L/atm

      - Air w/ 21% by volume O2

        - What’s the Cs of O2 in H2O @ 0C and 20C

Pi=ViVT=0.211.00=0.21 atm

          - 0C

CS=69.6 mgL−atm∗0.21=14.62 mgL

          - 20C

CS=39.3 mgL−atm∗0.21=8.25 mgL