- Solubility product

AaBb(s)  aAb+ + bBa- dissolution

Kso=Ab+aBa−bAaBb(s)Bb(s) goes to 1 due to activity of a solid

  - Activity of solids is  1

    - Actually derived from:

  - Must exercise cation

    - For example if CO3 solids are present must consider (may alter solubility)

      - Open or closed systems

      - Also other species may be present

      - Co-precipitation/adsorption

      - Complexation

  - Complexes - central atom with 2 to 9 other atoms, small molecules (referred to as ligands)

    - Ligands - also known as coordination compounds may be inorganic (e.g. Cl-, OH-, Fl-, NH3 etc.) or organic (e.g. EDTA, NTA, humic or folic acids)

R - CH - COO- + Cu2+  R - CH -CO

NH2				        O

H2N --- CU

    - Recall the reaction

aA + bB  cC +dD

If we start w/ A + B only

Equilibrium achieved

Conc.

time

C

A

D

B

    - If we start with C + D only (same equilibrium achieved)

A

C

B

D

Equilibrium achieved

Conc.

time

- Free Energy

  - Change in free energy of reaction = G

    - G = f(entropy, internal energy, and work)

G = H - TS

    - Where:

      - G = Gibbs free energy (kcal)

      - H = enthalpy (kcal) total energy of compound

      - T = (R) total part of energy not available to do work

      - S = entropy (kcal/R) internal energy

For a closed system at constant pressure and temperature "the criterion for equilibrium is the total free energy is at a minimum."

A + B only

present

G = - rate

GT,min = min free energy

GT

Total free

energy

Extent of reaction

      - If G = - rxn proceeds

      - If G = + rxn will not proceed

ΔG=iViGiproducts−iViGireactants

    - Where:

      - G = change in Gibbs free energy

      - GI = free energy per mole

    - If G = 0 then at equilibrium

    - G = overall standard free energy change

ΔG∘=iViΔGf∘products−iViΔGf∘reactants

    - For example:

CaCO3(s)+H+→Ca2++HCO32−

ΔG∘=ΔGf,HCO3−∘+ΔGf,Ca2+∘−ΔGf,CaCO3∘−ΔGf,H+∘

    - From tables:

ΔG∘=−140.31+−132.18−−269.78−0 ΔG∘=−2.71

    - It has also been shown that:

ΔG=ΔG∘+RTlnKeq at ΔG=0 ΔG∘=RTlnKeq

    - Can calculate Keq from free energy calculation

−2.71RT=lnKeq

    - True for half reactions

16SO42−+43H++e−→16S2++23H2OΔG∘=16ΔGf,S2+∘+23ΔGf,H2O∘−16ΔGf,SO42−∘−43ΔGf,H+∘ ΔG∘=−8.24kcal/mole can proceed

  - Enthalpy and temperature dependence of equilibrium constants

ΔH∘=ΔG∘+TΔS∘

    - Where:

      - H = change in enthalpy (heat taken up or released when A + B completely go to C + D)

      - S = change in entropy

ΔH∘=iΔHf∘products−iΔHf∘reactants

    - Van Hoff expression

dlnKdT=ΔH∘RT2

      - Over a limited range if H  f(T) then:

lnk1k2=ΔHR1T2−1T1 or lnk=ΔH∘RT+constant

    - Arrhenius equation:

lnK=lnA−EaRT

products

reactants

Ea = activation energy

H

Ext of Reaction

Avg.

Energy

Level

      - A + B  products or A + B  activated complex products

      - Ea is the addition energy needed to  activate the energy of transfer complex

      - Ea + H is given off at end of reaction

# Redox Potentials

  - Chemical energy can be used to create electrical energy and vice versa

  - Redox chemistry is important in natural and engineered systems

  - Oxidation – reduction reactions in WWTP are enzyme catalyzed

  - Driving force for these reactions is free energy of change, which can be directly related to electrical potentials

External circuit

Electrochemical cell – used to meas. the potential of half reactions

Sulfate can move across the barrier but not Cu2+ or Zn2+

Semi-permeable membrane

SO42-

Internal Circuit

Zn

Cu

Zn2+ + 2e- = Zn		Cu2+ +2e- = Cu

Cu2+

SO42-

  - If the electrodes are placed in solution over time zinc electrode is pitted, Cu electrode is deposited open. Electrical potential between the 2 can be measured and work is done.

    - From the energy change Cu2+ oxidizes Zn as follows

Zn + Cu2+ = Zn2+ + Cu

    - If Zn inc. Cu ions are at unit activity then a potential of 1.107 V would be measured

    - SO42- would migrate through the semi-permeable membrane to allow for electroneutrality

    - With this electrochemical cell the reaction would go to completion

  - Using a potentiometer a voltage opposing the voltage in the electrochemical cell is imposed to prevent the flow of current.  Concentration remains stable and a stable reading is obtained.

  - The cathode is the electrode at which the reduction takes place - cations receive electrons at this electrode

  - Anode is electrode in which oxidation takes place - electrons are removed

  - Electrons always flow from anode to cathode in external circuit

  - Can make electrochemical cells out of many materials

  - Potential for any oxidation - reduction rxn is measured against a standard H electrode

  - Hydrogen gas is bubbled into a solution to maintain a 1 atm PH2O, and [H+] in unit activity

  - Rxns are at 25C and at unit activity ( I2 except e- reduction)

12I2(aq)+e−→I−, EI2,I−∘12H2(g)→H++e−, EH2,H+∘12I2(aq)+12H2(g)→I−+H+, Ecell∘

Ecell∘=EI2, I−∘+EH2, H+∘=EI2, I−∘+0=EI2, I−∘

    - Under these conditions the value at:

EI, I−∘=standard electrical potential

    - If the value of E = + and all activities = 1 then the rxn proceeds spontaneously, because G and therefore G = -

    - Can writ the reactions in terms of G free energy

12I2(aq)+e−→I−

ΔG∘=ΔGf, I−∘−12ΔGf, I2(aq)∘−ΔGf, e−∘ΔG∘=−12.35−(.5∗3.93)−0=−14.32

12H2(g)→H++e−

ΔG∘=ΔGf, H+∘−12ΔGf, H2(g)∘+ΔGf, e−∘ ΔG∘=0−0+0=0

    - For the overall reaction:

ΔG∘=ΔGred∘+ΔGoxd∘=−14.32+0=−14.32 kcal, will proceed

    - E is related to G by:

ΔG∘=−nFE∘

      - n = number of electrons

      - F = Faradys constant

      - E = standard potential

    - Nernst equation:

E=E∘−RTnFlnproductsreactantsE=−ΔGnF E∘=ΔG∘nF

    - If

      - R = 8.314 V-coulombs/equiv

      - F = 96,500 coulombs/equiv

    - Then:

E=E∘−0.059nlog[C]c[D]d[A]a[B]b

    - Example: The ½ rxn for reduction of sulfate to sulfite:

SO42−+2H++2e−⇔SO32−+H2O

    - What is the standard potential EH of the rxn at 23C if it takes place in 10-3M SO32-/L and 10-4 moles SO42-/L at pH = 8

    - From table E = -0.04 V for the half rxn

    - Applying the Nernst Equation:

EH=E∘−0.059nlog[SO32−][H+]2[SO42−]EH=−0.04−0.0592log[10−3][10−8]2[10−4]EH=−0.50 Volt

ORD=Esys=E∘−0.059nlog[reduced sp][oxidized sp]