Lecture 4

Ce260/Spring 2000

- Analysis and Constituents in H2O

  - Standard methods for the examination of water and wastewater

  - EPA methods

  - Methods of Analysis

    - Titration – volume of titrant and concentration of titrant are important

      - e.g. 0.02 N H2SO4 used to titrate water to pH  4.5 to 4.8 w/100 mL of sample – alkalinity

      - Where:

[Alk]=[HCO3−]+[CO32−]+[OH−] [H2CO3] [HCO3−] [H2O] [CO2] [H2CO3] [CO2]

pH   4.8

ML 0.02 N H2SO4 added

  - Methods of Analysis

    - Ion specific electrodes

    - Atomic absorption spectrophotometry

      - Metals

      - Preparation needed

    - Ion chromatography

      - Anions and cations

    - High Performance Liquid Chromatography

      - Organic acids

    - Gas Chromatography

    - Mass Spectroscopy

    - UV/Vis Absorbance

    - Physical Tests

      - Total solids, TSS, TVS, and VSS

# Coarse | fine | colloidal | molecular

1000nm 100nm       1nm

      - Colloids – surface charge – characteristics govern color

      - Color

  - Determination of Oxygen Demand

    - e.g. VS is crude method

    - typically instrumental in O2 required to completely oxidize a compound biochemically are CO2 and H2O

    - importance of dissolved oxygen in systems natural and engineered

  - Theoretical Oxygen Demand (ThOD) stoichiometric

C6H12O6+6O2→6CO2+6H2O 180 192 264 108

ThODGlucose=192 mg O180 mg Glucose=1.07 mg Omg Glucose

C6H6+7.5O2→6CO2+3H2O

ThODBenzene=240 mg O78mg Benzene=3.08 mg Omg Benzene

Total CarbonGlucose=72 mg C180 mg Glucose=0.4 mg Cmg Glucose

Total CarbonBenzene=72 mg C78 mg Benzene=0.92 mg Cmg Benzene

    - If you had a solution of C6H12O6 @ 150 mg/L and C6H6 @ 15 mg/L:

      - Determine the ThOD of the solution on a per L basis

Glucose ThOD+Benzene ThOD=total ThOD

150mg GL∗1.07 mg Omg G+15mg BL∗3.08 mg Omg B=206.7mg OL

TC=150mg GL∗0.4 mg Cmg G+15mg BL∗0.92 mg Cmg B=73.8mg CL

  - Chemical Oxygen Demand

    - Amount of oxygen required to stabilize organic matter using a strong oxidant

    - Dichromate (Cr2O72-) is a typical chemical used (K2Cr2O7 is what is usually added)

    - Chromate reduction	Cr6+ Cr3+

    - Half reaction for chromate:

16Cr2O72−+73H++e−=13Cr3++76H2O

    - Overall reaction:

CnHaOb+cCr2O72−+fH+→dCr3++gCO2+eH2O

      - Where c, d, e, f, and g are stoichiometric coefficients

    - Through analysis and substitution:

CnHaOb+cCr2O72−+8cH+→2cCr3++nCO2+9+8c2H2O

Sample Problem

  - 10 mL of sample is diluted to 25-mL w/distilled H2O. Digested by standard COD procedure.  It was determined that 3.12/10-4 M Dichromate was used to oxidize the sample.  What is COD of the sample?  (dilution does not affect original sample organic matter)

    - 1 mole DC has 6 electron equivalents

    - 1 mole O2 has 4 electron equivalents

COD=3.12∗10−4moles DC10 ml∗6 equivmole DC8 g O2equiv∗1000mlL∗1000mgg=1498mg O2L

    - Interference's

      - If inorganic reducing agents are present then false positive

      - Test does not distinguish between biodegradable and non-biodegradable organic matter

  - BOD – Biochemical Oxygen Demand Test

    - BOD – amount of O required for biological degradation of organic matter under aerobic conditions at a standardized temperature and time of incubation.

- Recall: L = amount of organic matter expressed as O2 remaining at any time, t (Lt).

dLdt∝L so dLdt=−k, L

X=La−L=La(1−10−k, t)=La(1−e−k, t)

  - Streeter Phelps

    - TOC analysis, purge with acid:

CO32−, HCO3−, H2CO3→CO2(aq)→CO2(g)

    - In BOD bottle- e.g. of a mixed culture batch system