Lecture 10

CE 260 Spring/2000

# Coagulation and Flocculation

  - Coagulation is the destabilization of colloidal particles

    - Colloids – macromolecules and small particles that do not readily settlte out of suspension (e.g. clays, hydrous metal oxides, bacteria, viruses, proteins, pulp fibers = 1 um in size)

  - Stability of colloidal suspension

    - Varies greatly

    - Thermodynamically stable – are called reversible, can make unstable for short time but will become stable suspension again

    - Thermodynamically unstable – irreversible, (e.g. MOHx, viruses, and bacteria) we can aggregate these particles

  - Stability of particles is related to surface charge, the charge is a f(functional groups)

    - Acidic, basic, aphoteric OH groups on colloidal oxides –OH, silicates …

    - Localized –OH sites and O sharing in SiO4, AlO6, and AlO4 – clay zeolites

    - Ionizable surface sites on slightly soluble compounds

    - Acidic and basic functional groups on organic colloids, polysaccharides

  - Surface charge is related to ionization or protolysis of functional groups.

    - Therefore pH and ion content of aqueous phase

    - pH at which a colloid with an acidic functional group changes from uncharged to – charge is related to the pKa of those groups

    - e.g. carboxylic acid functional group on a protein has a pKa ~ 4 to 5, therefore a pH 4 colloid is uncharged and relatively unstable: @ pH 5 charges increase and stable suspension results (this is true for many microbial systems)

    - pH value at which the net surface charge is 0 is the zero point of charge (ZPC) therefore the zeta potential is zero (Er)

    - zeta potential is the net charge potential at a particular distance from the particle surface

  - Since colloidal dispersions are not charged so the primary charge on particle must be counterbalanced by the solution

  - Therefore an electrical double layer exist at the interface between the colloid and solution

    - The double layer has the charged particle and counterions of opposite charge equivalent to excess charge on the particle

    - Zeta potential expressed in following figures

| ## Composition | pHZPC |
| --- | --- |
| SiO2 (quartz) | 2.9 |
| -MnO2 (birnessite) | 2.2 |
| -Al(OH)3 | 5.0 |
| -Al2O3 | 8.5 |
| CaCO3 (calcite) | 9.5 |
| CuO | 9.5 |

    - Basically what this means is that salt in solution can mobilize ions

  - Common coagulants

    - Al2(SO4)3 alum sulfate (optimum pH 5.5-6.3)

    - Fe2(SO4)3 ferric sulfate (opt. pH 4.5-5.5)

    - Benefits

      - Adds cation 3+ valance state of metal

      - Hydroxides are very insoluble

    - Relative power of coagulants:

|  | + colloids | - colloids |
| --- | --- | --- |
| Al2(SO4)3 | 30 | >1000 |
| Fe2(SO4)3 | 30 | >1000 |
| NaCl | 1 | 1 |
| Na3PO4 | 1000 | 1 |

  - Chemistry of aluminum, iron salts and lime

    - Alum (pH decreases)

Al2(SO4)318H2O + 6H2O  2Al(OH)3(s) + 6H+ + 3SO42- + 18H2O

      - If alkalinity is present then

HCO3- +H+  H2CO3  CO2 +H2O

      - So pH will remain neutral, If lime is needed

Al2(SO4)318H2O + 3Ca(OH)2  2Al(OH)3(s) + CaSO4 + 6CO2 + 18H2O

    - Ferrous sulfate

Fe2SO47H2O + Ca(OH)2  Fe(OH)2 + CaSO4 +7H2O

4Fe(OH)2 + O2 +2H2O  4Fe(OH)3(s)

- Mixing and power dissipation – Newton’s law of viscosity for 1-D

τyx=−μdvxdy

  - Where:

    - yx = shear force per unit area in the x direction

    - dvx = velocity in the x direction

    - y = vertical direction

    -  = dynamic viscosity

If P=F⋅V then,τdvdy=ΔPΔV

    - P = power change

    - V = volume change

    - Applying Newton’s law of viscosity

μdvdy=ΔPΔV

    - In a fixed volume vessel, well mixed

ΔPΔV=PV

    - Defining G as the velocity gradient (dv/dy)

μG2=PV or G=PμV12P=ρgQhL

    - Q = volumetric flowrate

    - hL = head loss

    - Once P is calculated insert into G

    - G-td has been found to be correlated with mixers and flocculators

    - In 1973 Letterman developed an equation for the optimum Gtopt (optimum detention time) for values of G up to 5000s-1 used for water

| td, s | 20 | 30 | 40 | >40 |
| --- | --- | --- | --- | --- |
| G, s-1 | 1000 | 900 | 790 | 700 |

  - Mechanical and Pneumatic mixers

    - Impeller mixers are most efficient

      - Factors – geometry and basin impeller fluid characteristics, and $ power

      - Relationships

v∝ND A∝D2 P∝FDv=FDND

        - v = velocity of impeller

        - N = rpm (s)

        - D = impeller diameter

        - FD = drag force

        - P = power expenditure

    - Types of impellers Figure 13.5

CD=2FDρAv2=2FDvρAv3φ=PρN3D5Re=ρND2μ

      -  = power #, manufacturers list characteristics

Flocculators

D

vp = paddle

vf = fluid

1/2D

rL

b

  - b= length of paddle

  - Relative velocity

v=vp−vf=vp−kvp=vp1−kvp=2πNr60FD=12ρCDAv2

not dA=bdrdP=dFDv=12ρCDv3dAP=12ρCDb2πN601−k3riror3drP=1.44x10−4CDρbN1−k3ro4−ri4

  - Recommended design parameters for paddle flocculators

    - Tip speed 0.09 – 0.9 m/s

    - k = 0.25 w/o stators, 0-0.15 w/ stators

    - CD = 1.8 for flat blades

    - AB = 15 to 20% of tank cross-sectional area

    - Max depth approx. 5m

  - Go over example 13.2

hL=fLv2D2g